Simulation-Based Electromagnetic Analysis of TEM, TE and TM Propagation Modes in Microwave Transmission Structures

Code:

clc;

clear;

close all;

%% CONSTANTS

c = 3e8;

mu0 = 4*pi*1e-7;

eps0 = 8.854e-12;

mu = mu0;

eps = eps0;

v = 1/sqrt(mu*eps);

eta = sqrt(mu/eps);

%% RECTANGULAR WAVEGUIDE

a = 22.86e-3;

b = 10.16e-3;

f = 10e9;

w = 2*pi*f;

k = w/v;

lambda = v/f;

%% CUTOFF FREQUENCIES

fc_TE10 = c/(2*a);

fc_TE20 = c/a;

fc_TE01 = c/(2*b);

fc_TM11 = (c/2)*sqrt((1/a)^2+(1/b)^2);

fc = [fc_TE10 fc_TE20 fc_TE01 fc_TM11];

names = {'TE10','TE20','TE01','TM11'};

fprintf('\n--- MICROWAVE MODE ANALYSIS ---\n');

fprintf('Operating frequency = %.2f GHz\n',f/1e9);

fprintf('Wavelength = %.2f mm\n',lambda*1e3);

fprintf('Wave impedance = %.2f Ohms\n',eta);

fprintf('\nCutoff Frequencies:\n');

for i = 1:length(fc)

fprintf('%s = %.3f GHz\n',names{i},fc(i)/1e9);

end

%% PROPAGATION CONSTANTS

fprintf('\nPropagation Analysis:\n');

for i = 1:length(fc)

if f > fc(i)

beta = k*sqrt(1-(fc(i)/f)^2);

fprintf('%s: Propagating, beta = %.3f rad/m\n',...

names{i},beta);

else

alpha = k*sqrt((fc(i)/f)^2-1);

fprintf('%s: Below cutoff, alpha = %.3f Np/m\n',...

names{i},alpha);

end

end

%% FIELD CALCULATION GRID

Nx = 60;

Ny = 40;

x = linspace(0,a,Nx);

y = linspace(0,b,Ny);

[X,Y] = meshgrid(x,y);

%% TE10 FIELDS

Ey = sin(pi*X/a);

Hz = cos(pi*X/a);

%% TM11 FIELDS

Ez = sin(pi*X/a).*sin(pi*Y/b);

Ex_TM = (pi/a)*cos(pi*X/a).*sin(pi*Y/b);

Ey_TM = (pi/b)*sin(pi*X/a).*cos(pi*Y/b);

%% TEM FIELDS

d = 10e-3;

W = 30e-3;

V0 = 1;

xt = linspace(0,d,40);

yt = linspace(0,W,40);

[XT,YT] = meshgrid(xt,yt);

E0 = V0/d;

ETEM = E0*ones(size(XT));

HTEM = ETEM/eta;

%% FREQUENCY RANGE

freq = linspace(1e9,20e9,500);

%% COMBINED SUBPLOT FIGURE

figure('Name','Microwave Mode Analysis',...

'NumberTitle','off',...

'Color','w');

% 1. TE10 Electric Field

subplot(3,3,1);

surf(X*1000,Y*1000,Ey,'EdgeColor','none');

title('TE10 Electric Field');

xlabel('x (mm)');

ylabel('y (mm)');

zlabel('E_y');

colorbar;

view(45,30);

grid on;

% 2. TE10 Magnetic Field

subplot(3,3,2);

surf(X*1000,Y*1000,Hz,'EdgeColor','none');

title('TE10 Magnetic Field');

xlabel('x (mm)');

ylabel('y (mm)');

zlabel('H_z');

colorbar;

view(45,30);

grid on;

% 3. TM11 Longitudinal Electric Field

subplot(3,3,3);

surf(X*1000,Y*1000,Ez,'EdgeColor','none');

title('TM11 Longitudinal E Field');

xlabel('x (mm)');

ylabel('y (mm)');

zlabel('E_z');

colorbar;

view(45,30);

grid on;

% 4. TM11 Transverse Electric Field

subplot(3,3,4);

quiver(X*1000,Y*1000,Ex_TM,Ey_TM);

title('TM11 Transverse E Field');

xlabel('x (mm)');

ylabel('y (mm)');

axis equal tight;

grid on;

% 5. TEM Electric Field

subplot(3,3,5);

surf(XT*1000,YT*1000,ETEM,'EdgeColor','none');

title('TEM Electric Field');

xlabel('Separation (mm)');

ylabel('Width (mm)');

zlabel('E (V/m)');

colorbar;

view(45,30);

grid on;

% 6. TEM Magnetic Field

subplot(3,3,6);

surf(XT*1000,YT*1000,HTEM,'EdgeColor','none');

title('TEM Magnetic Field');

xlabel('Separation (mm)');

ylabel('Width (mm)');

zlabel('H (A/m)');

colorbar;

view(45,30);

grid on;

% 7. Cutoff Frequency Comparison

subplot(3,3,7);

bar(fc/1e9);

set(gca,'XTickLabel',names);

ylabel('Frequency (GHz)');

title('Cutoff Frequencies');

hold on;

yline(f/1e9,'r--','Operating Frequency');

grid on;

hold off;

% 8. Propagation Constant vs Frequency

subplot(3,3,8);

hold on;

for i = 1:length(fc)

beta = nan(size(freq));

idx = freq > fc(i);

beta(idx) = (2*pi*freq(idx)/v).* ...

sqrt(1-(fc(i)./freq(idx)).^2);

plot(freq/1e9,beta,'LineWidth',1.2);

end

xlabel('Frequency (GHz)');

ylabel('\beta (rad/m)');

title('Propagation Constant');

legend(names,'Location','northwest');

grid on;

hold off;

% 9. TE and TM Field Comparison

subplot(3,3,9);

imagesc(x*1000,y*1000,Ey);

set(gca,'YDir','normal');

xlabel('x (mm)');

ylabel('y (mm)');

title('TE10 Electric Field');

colorbar;

axis image;

sgtitle('TEM, TE AND TM MICROWAVE MODE ANALYSIS');

fprintf('\nSimulation completed successfully.\n');

OUTPUT:

--- MICROWAVE MODE ANALYSIS ---
Operating frequency = 10.00 GHz
Wavelength = 29.98 mm
Wave impedance = 376.73 Ohms

Cutoff Frequencies:
TE10 = 6.562 GHz
TE20 = 13.123 GHz
TE01 = 14.764 GHz
TM11 = 16.156 GHz

Propagation Analysis:
TE10: Propagating, beta = 158.154 rad/m

TE20: Below cutoff, alpha = 178.111 Np/m
TE01: Below cutoff, alpha = 227.635 Np/m
TM11: Below cutoff, alpha = 265.951 Np/m

Simulation completed successfully.